# C406 Cyclone IV Board - Pin Map and Findings

EP4CE6F17C8N FPGA with Winbond W9825G6KH SDR SDRAM (4M x 16 x 4 banks, 32 MB). Board marked “Altera Cyclone IV FPGA C406”, “Designed in Chengdu”, REV 1.1. Mapped 26 October 2026; no vendor schematic was available.

# 1. How the map was obtained

Pins routed to the two headers are printed on the bottom silkscreen and are not repeated here. Every other user I/O ball was loaded into a JTAG In-System Sources and Probes design, so each ball could be read or driven from the PC without knowing the board clock.

- Clock and key: found by watching which ball toggled, or changed when the key was pressed.

- LED: found by blinking each ball in turn.

- SDRAM: a multimeter on each SDRAM pin, answering high/low while the FPGA drove binary-coded patterns. Each result was confirmed by driving the found ball opposite to every other candidate.

- A self-test design then filled all 32 MB and read it back repeatedly at 50 MHz with zero errors, which confirms the SDRAM map as a whole.

- Finally, a PLL-based stress test ran the SDRAM at 100, 133, 150 and 166 MHz (its rated maximum) with zero errors - see section 6.

# 2. Clock, key and LED

| Function | FPGA ball | Port name | Notes |
| --- | --- | --- | --- |
| 50 MHz clock | M16 | clk_50m | Oscillator output on a dedicated clock input. Confirmed toggling. |
| User key | R8 | key_n | Active-low: reads 1 released, 0 pressed. Enable the FPGA weak pull-up - an external pull-up was not confirmed. |
| User LED | E10 | led | Active-high: drive 1 to light it. |

# 3. SDRAM layout, pin by pin

Top view of the 54-pin TSOP-II package with the pin-1 dot at the top left. Pins on the same row sit directly across from each other: the left side counts 1–27 downwards and the right side 28–54 upwards. Grey rows are power pins. The highlighted row (DQ12) needs a Quartus setting - see section 5.

| Pin | Signal | FPGA | Port | Pin | Signal | FPGA | Port |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | VDD | - |  | 54 | VSS | - |  |
| 2 | DQ0 | N16 | sdr_dq[0] | 53 | DQ15 | C14 | sdr_dq[15] |
| 3 | VDDQ | - |  | 52 | VSSQ | - |  |
| 4 | DQ1 | N15 | sdr_dq[1] | 51 | DQ14 | D12 | sdr_dq[14] |
| 5 | DQ2 | L15 | sdr_dq[2] | 50 | DQ13 | F15 | sdr_dq[13] |
| 6 | VSSQ | - |  | 49 | VDDQ | - |  |
| 7 | DQ3 | L16 | sdr_dq[3] | 48 | DQ12 | F16 | sdr_dq[12] |
| 8 | DQ4 | K15 | sdr_dq[4] | 47 | DQ11 | D16 | sdr_dq[11] |
| 9 | VDDQ | - |  | 46 | VSSQ | - |  |
| 10 | DQ5 | K16 | sdr_dq[5] | 45 | DQ10 | D15 | sdr_dq[10] |
| 11 | DQ6 | J15 | sdr_dq[6] | 44 | DQ9 | C16 | sdr_dq[9] |
| 12 | VSSQ | - |  | 43 | VDDQ | - |  |
| 13 | DQ7 | J16 | sdr_dq[7] | 42 | DQ8 | C15 | sdr_dq[8] |
| 14 | VDD | - |  | 41 | VSS | - |  |
| 15 | LDQM | G16 | sdr_dqm[0] | 40 | NC | - |  |
| 16 | WE# | G15 | sdr_we_n | 39 | UDQM | B16 | sdr_dqm[1] |
| 17 | CAS# | N14 | sdr_cas_n | 38 | CLK | A14 | sdr_clk |
| 18 | RAS# | P16 | sdr_ras_n | 37 | CKE | A15 | sdr_cke |
| 19 | CS# | P15 | sdr_cs_n | 36 | A12 | B14 | sdr_a[12] |
| 20 | BA0 | R16 | sdr_ba[0] | 35 | A11 | D11 | sdr_a[11] |
| 21 | BA1 | R14 | sdr_ba[1] | 34 | A9 | C11 | sdr_a[9] |
| 22 | A10/AP | P14 | sdr_a[10] | 33 | A8 | D14 | sdr_a[8] |
| 23 | A0 | N12 | sdr_a[0] | 32 | A7 | E11 | sdr_a[7] |
| 24 | A1 | L14 | sdr_a[1] | 31 | A6 | F13 | sdr_a[6] |
| 25 | A2 | L13 | sdr_a[2] | 30 | A5 | F14 | sdr_a[5] |
| 26 | A3 | J13 | sdr_a[3] | 29 | A4 | J14 | sdr_a[4] |
| 27 | VDD | - |  | 28 | VSS | - |  |

Power-pin assignments come from the standard JEDEC x16 TSOP-II pinout and were not measured; confirm them against the Winbond datasheet. All signal pins were measured.

## SDRAM signals by function

| SDRAM pin | Signal | FPGA ball | Port name | Notes |
| --- | --- | --- | --- | --- |
| 19 | CS# | P15 | sdr_cs_n | First pin found; hold high to keep the SDRAM idle |
| 18 | RAS# | P16 | sdr_ras_n | Verified |
| 17 | CAS# | N14 | sdr_cas_n | Verified |
| 16 | WE# | G15 | sdr_we_n | Verified |
| 37 | CKE | A15 | sdr_cke | Tied high in the test design |
| 38 | CLK | A14 | sdr_clk | Driven as inverted 50 MHz clock (DDIO) in the test design |
| 20 | BA0 | R16 | sdr_ba[0] | Verified |
| 21 | BA1 | R14 | sdr_ba[1] | Verified |
| 15 | LDQM | G16 | sdr_dqm[0] | Verified |
| 39 | UDQM | B16 | sdr_dqm[1] | Verified |
| 23 | A0 | N12 | sdr_a[0] | Verified |
| 24 | A1 | L14 | sdr_a[1] | Verified |
| 25 | A2 | L13 | sdr_a[2] | Verified |
| 26 | A3 | J13 | sdr_a[3] | Verified |
| 29 | A4 | J14 | sdr_a[4] | Verified |
| 30 | A5 | F14 | sdr_a[5] | Verified |
| 31 | A6 | F13 | sdr_a[6] | Verified |
| 32 | A7 | E11 | sdr_a[7] | Verified |
| 33 | A8 | D14 | sdr_a[8] | Verified |
| 34 | A9 | C11 | sdr_a[9] | Verified |
| 22 | A10/AP | P14 | sdr_a[10] | Also the auto-precharge flag on READ/WRITE |
| 35 | A11 | D11 | sdr_a[11] | Verified |
| 36 | A12 | B14 | sdr_a[12] | Verified |
| 2 | DQ0 | N16 | sdr_dq[0] | Verified |
| 4 | DQ1 | N15 | sdr_dq[1] | Verified |
| 5 | DQ2 | L15 | sdr_dq[2] | Verified |
| 7 | DQ3 | L16 | sdr_dq[3] | Verified |
| 8 | DQ4 | K15 | sdr_dq[4] | Verified |
| 10 | DQ5 | K16 | sdr_dq[5] | Verified |
| 11 | DQ6 | J15 | sdr_dq[6] | Verified |
| 13 | DQ7 | J16 | sdr_dq[7] | Verified |
| 42 | DQ8 | C15 | sdr_dq[8] | Verified |
| 44 | DQ9 | C16 | sdr_dq[9] | Verified |
| 45 | DQ10 | D15 | sdr_dq[10] | Verified |
| 47 | DQ11 | D16 | sdr_dq[11] | Verified |
| 48 | DQ12 | F16 | sdr_dq[12] | F16 is the nCEO dual-purpose pin - must be released as regular I/O |
| 50 | DQ13 | F15 | sdr_dq[13] | Verified |
| 51 | DQ14 | D12 | sdr_dq[14] | Verified |
| 53 | DQ15 | C14 | sdr_dq[15] | Verified |

# 4. Other balls

Behaviour observed on the balls that are neither on the headers nor used by the SDRAM, key or LED.

| Balls | Observed behaviour | Recommendation |
| --- | --- | --- |
| F5, L12 | Read high when released and stay high when driven low; something on the board holds them high. | Never drive low. Leave unused until the circuit is identified. |
| M15 | Dedicated clock input; floats and follows hand capacitance. | Not connected. Leave unassigned or add an external pull resistor. |
| E5 F6 F10 G11 J11 K6 K8 K12 L9 L10 L11 M12 | Read low and cannot be driven high. | Tied to ground. Never assign as outputs. |
| D4 F7 F11 J6 J12 K9 K10 K11 L6 M9 N13 | Read low with the internal pull-up, but go high when driven. | Weakly pulled low by something unknown. Leave unused. |
| M11 (top header) | Header pin that reads low with the internal pull-up, but goes high when driven. | Something weakly pulls it low; check before using it as an input. |
| B9 E6 E7 F2 F3 G2 G5 K5 L3 L4 L7 L8 N11 | Read high with the internal pull-up; nothing seen on them. | Probably unconnected. |
| C1, D2, H1, H2 | Configuration pins (ASDO, nCSO, DCLK, DATA0); kept reserved, not tested. | Leave reserved - they go to the configuration flash. |

# 5. Concerns

## Must be handled in every design

- SDRAM DQ12 is on F16, the nCEO pin. Set Device and Pin Options → Dual-Purpose Pins → nCEO to “Use as regular I/O”, or Quartus will not place sdr_dq[12]. c406_pins.tcl already does this.

- F5 and L12 must never be driven low. Something on the board holds them high, stronger than a 4 mA FPGA output. Driving them low causes a sustained fight. Their purpose is unknown.

- Twelve balls are tied to ground (E5 F6 F10 G11 J11 K6 K8 K12 L9 L10 L11 M12). Never use them as outputs.

- Key pull-up. R8 reads high when released only because the FPGA weak pull-up was enabled; no external pull-up was confirmed. Always enable WEAK_PULL_UP_RESISTOR on key_n.

- Unused pins. Keep “Reserve all unused pins” set to “As input tri-stated with weak pull-up”. The Quartus default on some versions drives unused pins to ground, which would fight F5 and L12.

## Unknown or unverified

- Power pins of the SDRAM were taken from the standard pinout, not measured.

- I/O bank voltages were not measured. 3.3-V LVTTL worked on every bank that was exercised, so 3.3 V VCCIO is likely throughout.

- Eleven balls are weakly pulled low by something unknown (D4 F7 F11 J6 J12 K9 K10 K11 L6 M9 N13), as is header pin M11, and F5/L12 are held high by something unknown. There may be circuitry on the top side not visible in the bottom-side photo.

- M15 is an unconnected, floating clock input.

- No vendor documentation. The vendor has been asked; the reply was pending. An AI-generated pinout found online was mostly wrong (only CKE = A15 and UDQM = B16 matched) and should not be used.

## Limits of the SDRAM verification

- Room temperature, one board. The timing windows in section 6 were measured at room temperature on this board only. At high temperature or low supply voltage they will be narrower; the recommended centre points leave about 2.5 ns of margin on each side at 166 MHz, but this was not measured under those conditions.

- The phase settings are tied to the clocking scheme. They apply to a PLL system clock with the SDRAM clock forwarded to A14 through a DDIO output register, as in sdram_stress. A different scheme (e.g. driving A14 directly from the PLL, or another PLL configuration) shifts the window; re-measure with stress.tcl. A14 is a regular I/O, not a dedicated PLL clock output.

- Datasheet margin. The measured windows are wider than the W9825G6KH-6 worst-case figures predict, which is normal for a typical part at room temperature. Do not rely on the window edges; use the centre points.

- Read capture timing is easy to get wrong. In the 50 MHz single-word test, sampling too late still passed because the undriven bus keeps its last value, so that test could pass with the wrong timing (use a 2-edge capture delay there). The burst stress test does not have this blind spot: consecutive words differ, so a late or early sample fails, as the failing phases in the sweep maps show.

- Byte masks and CKE were not exercised. LDQM/UDQM were held low and CKE high throughout; the pins themselves were verified with the meter, but byte-masked writes and power-down modes were not tested.

- Burst lengths 1 and 8 only. The 50 MHz test used single words and the stress test 8-word bursts, both with auto-precharge. Other burst lengths, full-page bursts and keeping rows open were not tested.

- SDRAM state after a clock-phase change. Stepping the SDRAM clock phase while running can corrupt the mode register. The stress test re-initialises the SDRAM before every fill and check; a design that changes the phase at run time must do the same.

# 6. SDRAM speed testing

A second test design (sdram_stress) drives the SDRAM from a PLL and can step the phase of the SDRAM clock at run time from the PC. The SDRAM runs at CAS latency 3 with 8-word bursts at the full clock rate, re-initialised before every test so that each test runs entirely at the phase being measured.

## Method

- Window sweep. At every PLL phase step through a full clock period, 64K words were written and read back with checkerboard (every data line switching every word) and random data, using each of four read-capture delays. This maps where both writes and reads work.

- Full suite at the centre of the widest window. Five patterns - address-unique (catches address-line faults), walking ones, checkerboard, random and solid - each also inverted, over all 32 MB; a retention test (fill, refresh only, check); and repeated runs of random data with different seeds.

- Long run at 166 MHz. A 120 s retention test and 200 full-memory random runs.

## Results

| Clock | Passing SDRAM-clock phase | Window | Centre (used) | Traffic tested | Errors |
| --- | --- | --- | --- | --- | --- |
| 100 MHz | 157°–345° (26 of 48 steps) | 54% / 5.4 ns | 255° / 7083 ps | 21 runs, 1.3 GB | 0 |
| 133 MHz | 90°–345° (18 of 24 steps) | 75% / 5.6 ns | 225° / 4688 ps | 21 runs, 1.3 GB | 0 |
| 150 MHz | 56°–349° (27 of 32 steps) | 84% / 5.6 ns | 203° / 3750 ps | 21 runs, 1.3 GB | 0 |
| 166 MHz | 45°–345° (21 of 24 steps) | 88% / 5.25 ns | 195° / 3250 ps | 232 runs, 14.8 GB | 0 |

Window = the range of SDRAM-clock phase (relative to the system clock) at which every test passed with read capture one cycle after the CAS latency. Centre = the phase used for the full suite. Each run writes and reads all 32 MB.

All four frequencies passed every test with zero errors. The FPGA design meets timing up to about 179 MHz in the slow corner, so the SDRAM's 166 MHz rating is the limit; it was not run out of specification. The window maps were identical before and after the long run.

## Recommended settings

- CAS latency 3 at all of these frequencies (CL2 is limited to 133 MHz by the -6 speed grade).

- SDRAM clock = PLL output with the static phase shift from the “Centre” column, forwarded to A14 through a DDIO output register; command, address and write data from the 0° system clock, in fast I/O registers.

- Capture read data on the system clock CL + 1 cycles after the READ command (capture delay 1 in sdram_stress).

- Refresh every 7.0 µs or sooner (8192 rows per 64 ms).

## 166 MHz window map

Each row is one 15° (250 ps) step of the SDRAM clock; each column a read-capture delay. A failing cell shows which data bits failed. The full suite ran at step 13 (bold).

| Phase step | Capture +0 | Capture +1 | Capture +2 | Capture +3 |
| --- | --- | --- | --- | --- |
| 0 (0°) | fail FFFF | fail FFFF | fail FFFF | fail FFFF |
| 1 (15°) | fail FFFF | fail FFFF | fail FFFF | fail FFFF |
| 2 (30°) | fail FFFF | fail FFFF | fail FFFF | fail FFFF |
| 3 (45°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 4 (60°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 5 (75°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 6 (90°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 7 (105°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 8 (120°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 9 (135°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 10 (150°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 11 (165°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 12 (180°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 13 (195°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 14 (210°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 15 (225°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 16 (240°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 17 (255°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 18 (270°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 19 (285°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 20 (300°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 21 (315°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 22 (330°) | fail FFFF | pass | fail FFFF | fail FFFF |
| 23 (345°) | fail FFFF | pass | fail FFFF | fail FFFF |

# 7. Project files

- board\c406_pins.tcl — all assignments above plus the required global settings; source it in new projects.

- sdram_test\ — the 50 MHz SDRAM self-test; quartus_stp -t status.tcl reads its pass and error counts over JTAG.

- sdram_stress\ — the PLL stress test: build.tcl builds one bitstream per frequency, stress.tcl runs the sweep and full suite, and result_*.txt hold the logs behind section 6.

- pinfinder\ — the JTAG pin-finder used for the mapping, with the raw results in found.txt.